DEMETER

IoT Plant & Aquarium Management System

Architecture & Proof of Concept Design

Version 1.0

March 2026

# Table of Contents

# 1. Executive Summary

Demeter is an IoT monitoring and management system designed for plant care and aquarium maintenance. The system uses ESP microcontrollers (ESP8266/ESP32) running MicroPython as sensor endpoints, communicating via the Constrained Application Protocol (CoAP, RFC 7252) over encrypted WiFi on a local area network.

A central management server built on FastAPI collects sensor telemetry, hosts a real-time dashboard, and exposes metrics for Prometheus scraping. AlertManager provides configurable alarms (soil moisture thresholds, aquarium temperature ranges) with email notifications. Grafana provides historical visualization, and Loki aggregates system logs.

The proof of concept focuses on establishing reliable CoAP communication including the Observe extension (RFC 7641) for event-driven notifications, demonstrating the full data pipeline from sensor to dashboard.

# 2. System Overview

## 2.1 Design Principles

- Lightweight protocol: CoAP is purpose-built for constrained devices, using UDP with minimal overhead

- Battery awareness: CoAP Non-Confirmable messages and Observe reduce radio-on time for battery-operated sensors

- LAN simplicity: All components reside on the same encrypted WiFi network; DTLS deferred to post-POC

- Observable events: Digital input changes push immediately via CoAP Observe, enabling real-time alerting

- Standards-based observability: Prometheus metrics, Grafana dashboards, Loki logging, AlertManager notifications

## 2.2 High-Level Architecture

The system follows a hub-and-spoke topology where ESP sensor nodes act as CoAP servers exposing observable resources, and the Demeter management server acts as a CoAP client observing those resources.

ESP Sensor Nodes (CoAP Servers) /sensors/soil_moisture /sensors/temperature /events/trigger │ CoAP Observe (UDP :5683) │ ▼ Demeter Server (CoAP Client + FastAPI) aiocoap Observer → In-Memory Store → FastAPI /api + /metrics │ HTTP Scrape :9090 │ HTTP :3000 │ HTTP :3100 │ ▼ Prometheus + AlertManager ¦ Grafana ¦ Loki

# 3. Component Design

## 3.1 ESP Sensor Nodes

Each ESP microcontroller runs MicroPython with an extended version of the microCoAPy library that adds CoAP Observe (RFC 7641) support. The node acts as a CoAP server, exposing sensor readings as observable resources.

### 3.1.1 Hardware Configuration

| Sensor Type | GPIO | Protocol | CoAP Resource |
| --- | --- | --- | --- |
| Soil Moisture | ADC (A0 / GPIO36) | Analog | /sensors/soil_moisture |
| Temperature | GPIO4 (OneWire) | DS18B20 | /sensors/temperature |
| Digital Input | GPIO5 | Digital / IRQ | /events/trigger |

### 3.1.2 CoAP Observe Extension

The CoAP Observe extension (RFC 7641) allows the Demeter server to register interest in a resource and receive notifications when its state changes. This is critical for two scenarios:

- Periodic telemetry: Soil moisture and temperature readings are pushed at configurable intervals (e.g., every 60 seconds) without the server polling each device

- Event-driven notifications: Digital input changes (e.g., a float switch, door sensor, or motion detector) trigger immediate notification to all registered observers

The Observe implementation adds Option Number 6 to microCoAPy. When a client sends a GET with Observe: 0, the server registers the observer. Subsequent state changes generate notifications with incrementing sequence numbers. The server deregisters observers on Observe: 1 or RST response.

### 3.1.3 Battery Optimization Strategy

- Non-Confirmable (NON) messages for periodic telemetry reduce retransmission overhead

- Configurable deep-sleep intervals between readings conserve battery on ESP32

- Observe notifications only fire on meaningful state changes (configurable delta thresholds)

- WiFi power management: light-sleep between transmissions on battery-operated nodes

## 3.2 Demeter Management Server

The server is a Python application combining aiocoap (CoAP client with Observe) and FastAPI (HTTP API, dashboard, and metrics). Both frameworks are asyncio-native, sharing a single event loop.

### 3.2.1 Server Components

| Component | Technology | Purpose |
| --- | --- | --- |
| CoAP Client | aiocoap | Observe ESP resources, collect telemetry |
| HTTP API | FastAPI | REST endpoints for dashboard, device management |
| Metrics Export | prometheus_client | Expose /metrics in Prometheus exposition format |
| Logging | python-logging-loki | Structured logging to Loki |
| Data Store | In-memory (POC) | Latest readings per device/resource |

### 3.2.2 Device Discovery & Registration

For the POC, devices are statically configured in a YAML file mapping device IPs to names and resource paths. Post-POC, CoAP resource discovery (/.well-known/core, RFC 6690) will enable automatic device enrollment.

## 3.3 Observability Stack

### 3.3.1 Metrics Pipeline

The Demeter server exposes a /metrics endpoint using the prometheus_client library. Each sensor reading is represented as a Prometheus gauge labeled by device name, sensor type, and resource path. Prometheus scrapes this endpoint at a configurable interval (default 15s).

### 3.3.2 Alerting

AlertManager rules are defined for critical thresholds:

- Soil moisture below configurable minimum (plant too dry)

- Soil moisture above configurable maximum (overwatering)

- Aquarium temperature outside safe range (e.g., 24–28°C for tropical fish)

- Device heartbeat timeout (sensor offline)

Alerts route to email via AlertManager SMTP configuration.

### 3.3.3 Visualization

Grafana connects to Prometheus as a data source and displays dashboards for real-time and historical sensor data. Loki provides a log exploration panel for debugging device communication issues.

# 4. Protocol Details

## 4.1 CoAP Message Flow

The following describes the message exchange between the Demeter server (client) and an ESP sensor node (server) for an Observe registration and subsequent notifications.

### 4.1.1 Observe Registration

| Step | Demeter (Client) | Dir | ESP (Server) | Type |
| --- | --- | --- | --- | --- |
| 1 | GET /sensors/temperature Observe: 0, Token: 0xAB | → |  | CON |
| 2 |  | ← | 2.05 Content, Observe: 1 Payload: 25.3, Token: 0xAB | ACK |
| 3 |  | ← | 2.05 Content, Observe: 2 Payload: 25.4, Token: 0xAB | NON |
| 4 |  | ← | 2.05 Content, Observe: 3 Payload: 25.5, Token: 0xAB | NON |

### 4.1.2 Event-Driven Notification (Digital Input)

When a digital input pin changes state (e.g., GPIO5 rising edge via hardware interrupt), the ESP immediately sends a notification to all registered observers with the new state. This provides sub-second response time for trigger events, unlike polled telemetry.

## 4.2 CoAP Resource Model

| Resource URI | Methods | Observe | Format | Description |
| --- | --- | --- | --- | --- |
| /sensors/soil_moisture | GET | Yes | JSON | Current soil moisture (0–100%) |
| /sensors/temperature | GET | Yes | JSON | Temperature in °C |
| /events/trigger | GET | Yes | JSON | Digital input state (0/1) |
| /config/interval | GET, PUT | No | JSON | Reporting interval in seconds |
| /.well-known/core | GET | No | Link | Resource discovery (post-POC) |

## 4.3 Payload Format

All sensor payloads use JSON with Content-Format 50 (application/json):

{ "device": "plant-01", "resource": "soil_moisture", "value": 42.5, "unit": "%", "ts": 1709827200 }

# 5. Implementation Phases

## 5.1 Phase 1 — ESP CoAP Endpoint

Goal: Functional ESP sensor node with CoAP server and Observe support.

- Extend microCoAPy with Observe option (Option Number 6) — observer registration, notification dispatch, and deregistration

- Implement sensor resource handlers for soil moisture (ADC), temperature (DS18B20), and digital input (GPIO with IRQ)

- Add periodic notification loop for telemetry resources and interrupt-driven notifications for digital input

- Test with coap-client (libcoap) or aiocoap CLI to validate Observe flow

Deliverables: Extended microCoAPy library, ESP sensor firmware, test scripts.

## 5.2 Phase 2 — Demeter Server

Goal: Server collecting sensor data via CoAP Observe and serving a dashboard with Prometheus metrics.

- Build aiocoap-based observer client that registers Observe on configured ESP resources

- Create FastAPI application with REST API endpoints and a simple HTML dashboard

- Expose /metrics endpoint with prometheus_client gauges for each sensor reading

- Integrate python-logging-loki for structured log shipping

Deliverables: Demeter server application, device configuration schema, API documentation.

## 5.3 Phase 3 — Observability Stack

Goal: Full monitoring pipeline with alerting and visualization.

- Deploy Prometheus with scrape configuration targeting Demeter /metrics

- Configure AlertManager rules for soil moisture, temperature, and device heartbeat thresholds with email routing

- Build Grafana dashboards: device overview, per-sensor time series, alert status panel

- Wire Loki as Grafana data source for log exploration

Deliverables: Docker Compose stack (Prometheus, AlertManager, Grafana, Loki), alert rules, dashboards.

# 6. Network & Security

## 6.1 POC Network Topology

All components operate on a single encrypted WiFi LAN (WPA2/WPA3). The network is assumed trusted for the proof of concept.

| Component | Port | Protocol | Direction |
| --- | --- | --- | --- |
| ESP Sensor Nodes | 5683/udp | CoAP | Server (inbound from Demeter) |
| Demeter Server | 8000/tcp | HTTP (FastAPI) | Inbound from browser/Grafana |
| Prometheus | 9090/tcp | HTTP | Scrapes Demeter /metrics |
| Grafana | 3000/tcp | HTTP | Queries Prometheus & Loki |
| Loki | 3100/tcp | HTTP | Receives logs from Demeter |
| AlertManager | 9093/tcp | HTTP | Receives alerts from Prometheus |

## 6.2 Post-POC Security Roadmap

- DTLS 1.2 encryption for CoAP communication (RFC 6347) using pre-shared keys initially

- Device authentication via CoAP token or DTLS client certificates

- HTTPS for all HTTP endpoints (FastAPI, Prometheus, Grafana)

- Network segmentation: IoT VLAN for sensor devices, management VLAN for server components

# 7. Technology Stack Summary

| Layer | Technology | Notes |
| --- | --- | --- |
| MCU | ESP8266 / ESP32 | ESP32 preferred for ADC quality and deep sleep |
| MCU Runtime | MicroPython | Rapid prototyping, OTA update friendly |
| CoAP (MCU) | microCoAPy (extended) | Fork with Observe support added |
| CoAP (Server) | aiocoap | Mature async CoAP with native Observe |
| HTTP API | FastAPI + Uvicorn | Async HTTP, auto-generated OpenAPI docs |
| Metrics | Prometheus + prometheus_client | Time-series storage and scraping |
| Alerting | AlertManager | Threshold alerts with email routing |
| Visualization | Grafana | Dashboards for Prometheus and Loki |
| Logging | Loki + python-logging-loki | Lightweight log aggregation |
| Transport | WiFi (WPA2/WPA3) | LAN only for POC; DTLS post-POC |

# 8. Risks & Mitigations

| Risk | Impact | Likelihood | Mitigation |
| --- | --- | --- | --- |
| UDP packet loss | Missed readings | Low (LAN) | CON messages for critical data; heartbeat monitoring |
| ESP memory constraints | Observer limit | Medium | Cap observers at 4; compact data structures |
| WiFi reconnection delays | Data gaps | Medium | Auto-reconnect with exponential backoff; local buffering |
| No encryption (POC) | Data exposure | Low (LAN) | DTLS planned for post-POC; WPA2 provides link-layer encryption |
| Battery drain | Frequent recharging | Medium | Deep sleep; NON messages; configurable intervals |

# 9. Future Enhancements

- CoAP Block-wise Transfer (RFC 7959) for firmware OTA updates over CoAP

- CoAP Resource Discovery (RFC 6690) with /.well-known/core for automatic device enrollment

- DTLS 1.2 encryption with pre-shared keys, progressing to certificate-based authentication

- Multi-gateway support for larger deployments across multiple LAN segments

- Time-series database (InfluxDB or TimescaleDB) replacing Prometheus for high-cardinality long-term storage

- Mobile push notifications via AlertManager webhooks to Pushover or ntfy.sh

- Actuator control: CoAP PUT to ESP resources for relay switching (pumps, heaters, lights)